| Roll No | 22SW040 Section_01 |
| --- | --- |
| Name | Farooque Sajjad |
| Subject | CN Practical (LAB_10) |
| Teacher | Ma’am Aisha |

ANSWERING THE QUESTIONS MENTIONED IN THE HANDOUT

Why is the interface Serial0 changed state to down?

The interface Serial0 changes state to "down" because it does not detect a signal from the other end of the serial link. This could be due to either the cable being disconnected, a misconfiguration, or the interface on the other connected router being down.

Are all the necessary interfaces up?

Yes, all the necessary interfaces are up seeing the screenshots below

What networks are displayed on Router3?

Router3 displays the following networks:

172.16.0.0 (directly connected to Serial0)

192.168.3.0 (directly connected to Ethernet0)

192.168.0.0, 192.168.2.0, and 192.168.1.0 (learned via RIP)

Which network is directly connected to the Ethernet port?

The 192.168.3.0 network is directly connected to Router3’s Ethernet0 interface.

Why are there three network entries on Router2?

Router2 has three network entries because it has interfaces on three different subnets:

192.168.0.0 (connected to Serial0)

172.16.0.0 (connected to Serial1)

192.168.2.0 (connected to Ethernet0)

These networks are included in the RIP configuration to advertise them to other routers.

List the shortest listed route in the routing table of R2.

The shortest listed route in Router2’s routing table would be the directly connected networks with a metric of 0, such as 192.168.0.0, 172.16.0.0, and 192.168.2.0.

What is the administrative distance, and which protocol is used?

The administrative distance for RIP is 120, and the routing protocol used here is RIP (Routing Information Protocol).

TASK

First drag and drop 3 routers and pcs

Then adding NM-1E and WIC-IT to each of the router

Now connecting routers and pcs

Assigning the IP addresses to the connected pcs

Now configuring the Serial and Fast Ethernet Interfaces for all  the Routers

All the connection becomes green (all interfaces are up)

Running the command (sh ip route and s hip int brief) on both the Routers

Now Doing RIP Routing between all the connected Routers

Again, Running the Command (s hip route) to reflect the Dynamic routing

Now Running the ping command from PC0, PC1, and PC2 for all the connections